Flying Orders Addendum - PA 28 G-BSSC and G-SEJW

These additions to our flying orders apply to the aircraft above during the period we hire them.

They should be read in conjunction with, and additional to our other Flying Orders.

1 Checkout

Before flying either of the aircraft above you must have had a checkout with an instructor, the period will be determined by them based on your flying and familiarity with avionics.

Duel G5 requires a sign off in your logbooks.

2 Refuelling

Refuel the aircraft before flight, uplifting the amount you intend to use.  Otherwise, all the normal operations apply.  Complete the fuel log before you go flying.

Do not refill after flying, due to reduced load capability.

A PA 28 holds 182L useable when both tanks full and 130L to ‘tabs’, but check POH

This is to enable those pilots that fly with two or more passengers to do so within the weight limitations of the aircraft.

3 Weight & Balance

If flying with more than 1 passenger we will expect you to carry out a W&B for the flight and produce it if required.

4 Flying Time Recording

The new Flight Log (now on the Notice page of Shlott) will require you to record take-off and landing times to exact minutes and that is what you will be charged for, as well as block time (brakes off/brakes on rounded to 5 mins) because that is what goes in your logbook.  Also, the tacho readings in SC.

This ensures you have time needed to carry out proper warmup, taxi slowly to the runway, and all the ground/runup checks to enable flight to be conducted safely.  On landing, record that time, which again then gives time to taxi slowly back to hangar and shutdown in an orderly way.  You will need to record the block times for your logbook (rounded).

Please check all entries are correct before you lockup the clubhouse.

5 Propellor Monitoring

We are going to ask you to take pictures of both blades before and after flight as evidence they did not sustain damage while you were operating the aircraft.  Given the way in which we will be recording flight time there is no excuse for taxiing at any speed above walking pace.  Keep the yoke held fully back at all times when taxiing to lift prop as high as possible.

Our WhatsApp group has been modified and is now a ‘community’ with a new logo, to keep/post photos of the prop from your phone, plus all the other things we post.

Repairs/redressing costs may sit with the pilot flying, therefore it is in your interests to ensure you have photos.  Add a note to your checklist perhaps?

6 Hangar Parking

We may have to review the boxes on the floor but initially line up the PA28 nosewheel with the centre nosewheel box and the mains in line with the other boxes, if they currently don’t fit.

Please be aware that the wingspan of a PA28 is 2m more than the Robin DR400

There will no doubt be other things we need to update as we become familiar with SC.

01/05/2026